PIR датчик

Код на Python

# MBTechWorks.com 2017

# Use an HC-SR501 PIR to detect motion (infrared)

#!/usr/bin/python

import RPi.GPIO as GPIO

import time

GPIO.setmode(GPIO.BOARD) #Set GPIO to pin numbering

pir = 8 #Assign pin 8 to PIR

led = 10 #Assign pin 10 to LED

GPIO.setup(pir, GPIO.IN) #Setup GPIO pin PIR as input

GPIO.setup(led, GPIO.OUT) #Setup GPIO pin for LED as output

print ("Sensor initializing . . .")

time.sleep(2) #Give sensor time to startup

print ("Active")

print ("Press Ctrl+c to end program")

try:

while True:

if GPIO.input(pir) == True: #If PIR pin goes high, motion is detected

print ("Motion Detected!")

GPIO.output(led, True) #Turn on LED

time.sleep(4) #Keep LED on for 4 seconds

GPIO.output(led, False) #Turn off LED

time.sleep(0.1)

except KeyboardInterrupt: #Ctrl+c

pass #Do nothing, continue to finally

finally:

GPIO.output(led, False) #Turn off LED in case left on

GPIO.cleanup() #reset all GPIO

print ("Program ended")

Установка Pygame

sudo apt-get install python-pip

sudo pip install pygame

Обновление системы

apt-get update

apt-get upgrade

Запуск аудиофайла

import pygame

pygame.mixer.init()

pygame.mixer.music.load("myFile.wav")

pygame.mixer.music.play()

while pygame.mixer.music.get_busy() == True:

continue